GMS-V1X02 PARAMETER LIST

Content

1	Revisions	1

2	Explanations	1

3	Units	2

4	BRM Parameters	2

1.1	Rotor position	2

1.2	Refiner vibration	2

1.3	Touch vibration	2

1.4	Control motor	3

1.5	Hydraulic pressure A	3

1.6	Hydraulic pressure B	3

1.7	Temp fr. bearing	3

1.8	Temp to bearing	3

1.9	Main motor power	4

1.10	Analog input	4

1.11	FeedGuard func.	4

1.12	Rotor Move Ctrl.	4

1.13	Rotor movement adjustment	5

1.14	Digital In/Out	5

1.15	Analog out module	5

5	CONTACT	5

Revisions

Feb. 18, 2020/BL	First release for revision 1.25.

Explanations

Yes	The parameter is used and should contain a valid value.

No	The parameter is not used and the value is of no importance.

(*)	The parameter can only be read.

(!)	The parameter is calibrated, check the calibration manual

Units

Unit	Description	Serie no.	HW Revision	SW-revision

GMS-V1X02	System	-	-	-

BRM-DM1	Basic Refiner Module	1XX	2.11	1.25

BRM Parameters

BRM GMS System	7: GMS-V1X02		yes

BRM CAN Node no.	1		yes

BRM Ind. Bus type.	1		yes

BRM Serial no.	1XX		(*)

BRM HW Rev. no.	2.11		(*)

BRM DCS Clock	X … X		yes

BRM Test func.	0		yes

BRM Date	xxxx…		no

Rotor position

POM = XX.XX	-		(*)

POM Off/On/On+Al.	On+Alarm		yes

POM Direction	0	-	yes

POM Ratio	1.00	-	yes

POM Al.lim 1 HH (Limit 1, HH, hi-hi)	-	mm	no

POM Al.lim 2 H (Limit 2, H, high)	-	mm	no

POM Al.lim 3 L (Limit 3, L, low)	-	mm	no

POM Zero cal.	= XX.XX	mm	yes (!)

POM Span cal.	= XX.XX	mm	yes (!)

POM Work mode	Dist./Speed		yes (!)

Refiner vibration

VIM = X.X mm/s	X.X	mm/s	(*)

VIM Off/On/On+Al.	On+Alarm		yes

VIM Al.lim 1 HH (Alarm limit, Limit 1, HH, hi-hi)	-	mm/s	no

VIM Al.lim 2 H (Alarm limit, Limit 2, HH, hi)	-	mm/s	no

Touch vibration

TVD = X %	X	%	(*)

TVD Off/On/On+Al.	On+Alarm		yes

TVD Sense idle	80	%	yes

TVD Sense prod	20	%	yes

TVD Al.lim 1 Hg2 (Alarm limit, Limit 1, idle, hi-hi)	-	%	no

TVD Al.lim 2 Hg1 (Alarm limit, Limit 2, idle, hi)	-	%	no

TVD Al.lim 3 Lg2 (Alarm limit, Limit 3, prod, hi-hi)	-	%	no

TVD Al.lim 4 Lg1 (Alarm limit, Limit 4, prod,,hi)	-	%	no

TVD Pro.adj.sen.	50	%	yes

Control motor

CMD = Status	-		(*)

CMD Off/On/On+Al.	On		yes

CMD Screw pitch	1	mm/varv	yes

CMD Low speed	0.05	mm/s	yes

CMD High speed	0.25	mm/s	yes

CMD Direction	0		yes

CMD Drive current	2.00	A	yes

CMD Hold current	1.00	A	yes

CMD Min hold cu.	0.05	A	yes

CMD Run current	X.X	A	(*)

CMD Status hold one of the following messages: Idle, Open gap Lo, Close gap Lo, Open gap Hi, Close gap Hi. Lo and Hi means low or high speed.

Hydraulic pressure A

HPA = 0 ton	0		(*)

HPA Off/On/On+Al.	Off		yes

HPA Al.lim 1 HH (Alarm limit, Limit 1, hi-hi)	-	ton	no

HPA Al.lim 2 H (Alarm limit, Limit 1, hi)	-	ton	no

HPA Range	- 	ton	no

HPA Zero cal.	= XX.XX	ton	no (!)

HPA Span cal.	= XX.XX	ton	no (!)

Hydraulic pressure B

HPB = 0 ton	0		(*)

HPB Off/On/On+Al.	Off		yes

HPB Al.lim 1 HH (Alarm limit, Limit 1, hi-hi)	-	ton	no

HPB Al.lim 2 H (Alarm limit, Limit 2, hi)	-	ton	no

HPB Range	- 	ton	no

HPB Zero cal.	= XX.XX	ton	no (!)

HPB Span cal.	= XX.XX	ton	no (!)

Temp fr. bearing

OT1 = 0	0	ºC	(*)

OT1 Off/On/On+Al.	Off		yes

OT1 Al.lim 1 HH (Alarm limit, Limit 1, hi-hi)	-	ºC	no

OT1 Al.lim 2 H (Alarm limit, Limit 2, hi)	-	ºC	no

OT1 Zero cal.	= XX.XX	ºC	no (!)

OT1 Span cal.	= XX.XX	ºC	no (!)

Temp to bearing

OT2 = 0	0	ºC	(*)

OT2 Off/On/On+Al.	Off		yes

OT2 Al.lim 1 HH (Alarm limit, Limit 1, hi-hi)	-	ºC	no

OT2 Al.lim 2 H (Alarm limit, Limit 2, hi)	-	ºC	no

OT2 Zero cal.	= XX.XX	ºC	no (!)

OT2 Span cal.	= XX.XX	ºC	no (!)

Main motor power

MPM = 0	0	MW	(*)

MPM Off/On/On+Al.	Off		yes

MPM Al.lim 1 LL (Alarm limit, Limit 1, low-Low)	-	MW	no

MPM Al.lim 2 L (Alarm limit, Limit 2, low)	-	MW	no

MPM Al.lim 3 Float (Alarm limit, Limit 3, Floating)	-	MW	no

MPM Al.lim 3 filt. (Alarm limit, Limit 3, Filtrer)	-	MW	no

MPM Range	-	MW	no

MPM Type	-		no

MPM Zero cal.	= XX.XX	MW	no (!)

MPM Span cal.	= XX.XX	MW	no (!)

Analog input

AIN = 0	0	-	(*)

AIN Off/On/On+Al.	Off		yes

AIN Al.lim 1 LL (Alarm limit, Limit 1, low-Low)	-	-	no

AIN Al.lim 2 L (Alarm limit, Limit 2, low)	-	-	no

AIN Type	-		no

AIN Range	-	-	no

AIN Zero cal.	= XX.XX	-	no (!)

AIN Span cal.	= XX.XX	-	no (!)

FeedGuard func.

FG = Activated	-	-	(*)

FG Result	Dist = XX.X	mm--	(*)

FG Type	2	-	yes

FG Piston length	1.0	mm	yes

FG Safe dist.	1.0	mm	yes

FG Timeout	10	s	yes

FG Piston delay	0.5	s	yes

FG CMD Speed	0.25	mm/s	yes

FG Chk.max pos	39.00	mm	yes

Rotor Move Ctrl.

RMC = On Dynamic op.	-		(*)

RMC Function	Dynamic op.	-	yes

RMC Prod.pos.	XX.XX	mm	yes

RMC Start offset	1.00	mm	yes

RMC Set prod.pos.	0.00	mm	yes

RMC Power limit	1.00 	MW	yes

RMC Time limit	1.0	min	yes

RMC Min prod.pos	10.00	mm	yes

RMC Max prod.pos	40.00	mm	yes

Rotor movement adjustment

RMA = PLC ctrl.cal	-	-	(*)

RMA Touchp.func.	PLC ctrl.	-	yes

RMA Touchp.level	40	%	yes

RMA Touchp.time	1.0	s	yes

RMA Touchp. pos.	XX.XX	mm	(*)

RMA Wear setting	1	-	yes

RMA Wear offset	XX.XX (#)	mm	yes

# The value is automatically set but can also be changed.

RMA Status	State=0xXX		(*)

Digital In/Out

DIO Input= XX	X		(*)

DIO Out Status= XX	X		(*)

DIO In Configure	0		yes

DIO Out Configure	0		yes

Analog out module

AOM = Disabled	-	-	(*)

AOM Mod.1 Chan.1	---		yes

AOM Mod.1 Chan.2	---		yes

AOM Mod.1 Chan.3	---		yes

AOM Mod.1 Chan.4	---		yes

AOM Mod.2 Chan.1	---		yes

AOM Mod.2 Chan.2	---		yes

AOM Mod.2 Chan.3	---		yes

AOM Mod.2 Chan.4	---		yes

CONTACT

Sales, development, production and service:

Dametric AB

Jägerhorns Väg 19, SE 141 75 Kungens Kurva, Sweden

Phone:  +46-8 556 477 00	Telefax: +46-8 556 477 29

E-Mail: service@dametric.se	Website: www.dametric.se